INSULATED ROLLER SHUTTERS

SPECIFICATION 3-8

Curtain

Constructed from 100mm flat galvanised insulated steel laths with endlocks fitted to alternate laths to prevent lateral movement.

Bottom Rail

Fabricated from the bottom lath with a galvanised steel trim section and fitted with a rubber weatherstrip.

## Guides

The side guides are formed from galvanised steel with twin brush strips and prepared for fixing to the opening structure by means of continuous fixing angles.

## Endplates

Fabricated from mild steel of adequate thickness relative to door size and supplied with angles for fixing to the structure.

## Roller

The roller is manufactured from mild steel tube of sufficient diameter and wall thickness to resist deflection and is mounted in bearings or cups on each end plate.

Finish

Galvanised as standard.  A factory finish is available which comprises a white plastisol internal face (coil side) with the external face polyester powder coated to a standard RAL colour.

## Weight

Approximately 45 kgs per m².

Operation

Electrical operation is by means of a direct drive 3 phase geared motor mounted on the outside of one endplate.  The motor has an inherent safety brake thus meeting all relevant Health & Safety requirements and the standard unit is pre-wired obviating the need for a site electrician.

Controls

The standard method of control is by means of internal wall-mounted push button control.  Other types of control including keyswitches etc. are available at extra cost.

Casings

Coil casings fabricated from galvanised steel sheet are available as an optional extra.

To specify state:

Insulated Roller Shutters shall be by

Bolton Gate Company, Waterloo Street, Bolton BL1 2SP, UK Tel: 01204 871001

E-mail: sales@boltongate.co.uk Web: www.boltongate.co.uk

Issue Date: April 2018