ROLLER SHUTTERS

(1 PHASE TUBULAR MOTOR)

SPECIFICATION 3-1

Curtain

Constructed from 76mm curved section galvanised steel laths with steel endlocks fitted to alternate laths to prevent lateral movement. The bottom rail is a galvanised steel inverted “tee” section.

## Guides

The side guides are formed from galvanised steel channels and prepared for fixing to the opening structure by means of continuous fixing angles.

## Endplates

Fabricated from mild steel of adequate thickness relative to door size and supplied with angles for fixing to the structure.

## Roller

The roller is manufactured from mild steel tube of sufficient diameter and wall thickness to resist deflection and is mounted in bearings or cups on each end plate.

Finish

The majority of parts are galvanised with other parts prime painted.  They are also available in stainless steel and polyester powder coated in a range of standard RAL colours at extra cost.

## Weight

Approximately 35 kgs per m².

Operation

Electrical operation is by means of a single phase tubular motor.  A safety brake is fitted to all shutters where personnel could pass underneath thus meeting all relevant Health & Safety requirements. The client should provide a fused spur within 500mm of the motor position.

Controls

The standard method of control is by means of internal wall-mounted push buttons.  Other types of control including keyswitches and group controls are available at extra cost.

Casings

Coil casings fabricated from galvanised steel sheet are available as an optional extra.

To specify state:

Roller Shutters shall be by

Bolton Gate Company, Waterloo Street, Bolton BL1 2SP, UK Tel: 01204 871001

E-mail: sales@boltongate.co.uk Web: www.boltongate.co.uk

Issue Date: April 2018