ESCALATOR FIRE SHUTTERS

SPECIFICATION 4-1

Fire Resistance

Tested for four hours and meeting the requirements of BS 476 Part 22 1987.

Curtain

Constructed from curved section galvanised steel laths.

Leading Edge

Formed from two steel angles to form a “tee” section.

## Guides

The side guides are formed from mild steel channels and are prepared for fixing to the opening structure by means of continuous steel angles.  The standard guide size is 110mm high x 44mm high.

## Casing

The casing is manufactured from galvanised steel and incorporates the barrel onto which the curtain coils.  The standard casing is 1145mm wide x 1275mm high.

Finish

Galvanised or primed as standard.  A polyester powder coated finish in a range of standard RAL colours is available at extra cost.

## Weight

Casing: 450 kgs per linear metre

Curtain: 24 kgs per m²

Guides: 30 kgs per linear metre

Operation

Operation is by means of a 400 volt 3 phase geared motor mounted at one end of the casing.  A control panel is provided adjacent to the motor.  A maintained and protected AC supply is required, terminated in an isolator within one metre of the motor. Local control is by means of keyswitch.

Fire Activation

In a fire condition, activation is initiated by an incoming signal from the fire alarm system.

To specify state:

Escalator Fire Shutters shall be by

Bolton Gate Company, Waterloo Street, Bolton BL1 2SP, UK Tel: 01204 871001

E-mail: sales@boltongate.co.uk Web: www.boltongate.co.uk.

Issue Date: April 2018