EUROFOLD FOLDING DOORS

(ELECTRIC OPERATION)

SPECIFICATION 1-3

Gate Frame

Constructed from double channel rolled pickets interconnected by galvanised steel lattice bars.  Ball bearing pulleys are attached at the top and zinc-plated guide pieces at the bottom.

Leading Edge

Galvanised rolled steel section reinforced for handles, locks and drive chain connection.

## Leaves

Formed from 305mm x 1.2mm galvanised steel and ribbed for strength.  Front hinge strips are attached and the interlocked assembly provides a weather tight installation.

## Top Track

Galvanised steel rolled section forming a box-type track 90mm deep.

## Bottom Track

Type B28 galvanised steel “m” section.

Finish

The majority of parts are galvanised with other parts prime painted.  Polyester powder coating to a range of standard RAL colours is available at extra cost.

## Weight

Approximately 32 kgs per m².

Operation

Electrical operation is by means of a direct drive 3 phase geared motor mounted on the top track.  Manual operation in the event of power failure is by means of hand chain accessible from low level.

Controls

The standard controls are by means of open / close / stop push buttons fitted adjacent to the door. Other types of control are available at extra cost.

To specify state:

Eurofold Folding Doors shall be by

Bolton Gate Company, Waterloo Street, Bolton BL1 2SP, UK Tel: 01204 871001

E-mail: sales@boltongate.co.uk Web: www.boltongate.co.uk

Issue Date: April 2018