EUROFOLD MANUAL FOLDING DOORS

SPECIFICATION 1-3

Gate Frame

Constructed from double channel rolled pickets interconnected by galvanised steel lattice bars.  Ball bearing pulleys are attached at the top and zinc-plated guide pieces at the bottom.

Leading Edge

Galvanised rolled steel section reinforced for handles and locks.

## Leaves

Formed from 305 x 1.2mm galvanised steel and ribbed for strength.  Front hinge strips are attached and the interlocked assembly provides a weather tight installation.

## Top Track

Galvanised steel rolled section forming a box-type track 90mm deep.

## Bottom Track

Type B28 galvanised steel “m” section.

Finish

The majority of parts are galvanised with other parts prime painted.  Polyester powder coating to a range of standard RAL colours is available at extra cost.

## Weight

Approximately 32 kgs per m².

Operation

Manual operation via bow handles attached to each leading edge.

Locks

Locking is available via a five-lever deadlock mortised into the leading edge.

To specify state:

Eurofold Manual Folding Doors shall be by

Bolton Gate Company, Waterloo Street, Bolton BL1 2SP, UK Tel: 01204 871001

E-mail: sales@boltongate.co.uk Web: www.boltongate.co.uk

Issue Date: April 2018