EUROFOLD LIFT LANDING DOORS

SPECIFICATION 1-14

Fire Resistance

Tested to EN 81-58 and achieved an E120 fire rating.

Gate Frame

Constructed from double channel rolled pickets interconnected by galvanised steel lattice bars.  Ball bearing pulleys are attached at the top and zinc-plated guide pieces at the bottom.

Leading Edge

Galvanised rolled steel section reinforced for handles and locks.

## Leaves

Formed from 152 or 230mm x 1.2mm galvanised steel and interconnected by front and back hinge strips.

Vision Panel

A 229 x 89mm fire resisting glass vision panel is available for incorporation into the first leaf.

## Top Track

Galvanised steel rolled section forming a box-type track 90mm deep.

## Bottom Track

Type B25 26 x 25mm channel section.

Finish

The majority of parts are galvanised with other parts prime painted.  Polyester powder coating to a range of standard RAL colours and stainless steel are available at extra cost.

## Weight

Approximately 36 kgs per m².

Operation

Manual operation via bow handles attached to each leading edge.

Ironmongery

Holdfast magnets and VL10 gatelocks are available at extra cost.

Frames

Three or four-sided fire rated steel frames are available at extra cost.

To specify state:

Eurofold Lift Landing Doors shall be by

Bolton Gate Company, Waterloo Street, Bolton BL1 2SP, UK Tel: 01204 871001

E-mail: sales@boltongate.co.uk Web: www.boltongate.co.uk

Issue Date: April 2018