FIREROLL E240 FIRE RESISTING ROLLER SHUTTERS

SPECIFICATION 3-5

Fire Resistance

Tested to BS 476 Part 22 and EN 1634-1 and approved for fire ratings up to four hours (E240 rating in accordance with EN 13501-2).

Curtain

Constructed from 76mm curved section galvanised steel laths with steel endlocks fitted to alternate laths to prevent lateral movement.

Bottom Rail

Galvanised steel inverted “tee” section.

## Guides

The side guides are formed from galvanised steel channels and prepared for fixing to the opening structure by means of fixing angles.

## Endplates

Fabricated from mild steel of adequate thickness relative to door size and supplied with angles for fixing to the structure.

## Roller

The roller is manufactured from mild steel tube of sufficient diameter and wall thickness to resist deflection and is mounted in bearings or cups on each end plate. A galvanised steel casing is supplied as standard.

Finish

The majority of parts are galvanised with other parts prime painted.  They are also available in stainless steel and polyester powder coated in a range of standard RAL colours at extra cost.

## Weight

Approximately 50 kgs per m².

Operation

Electrical operation is by means of a single or three phase Speedsafe motor mounted on one endplate.  The standard unit is pre-wired obviating the need for a site electrician. Standard controls are by means of open / close / stop push buttons fitted adjacent to the shutter.

Fire Activation

In a fire condition, closure is activated by a fusible link.  If activation is required from the fire alarm system, an auto re-set solenoid or magnet is available at extra cost.  Descent is always at a controlled speed.

To specify state:

Fireroll E240 Fire Resisting Roller Shutters shall be by

Bolton Gate Company, Waterloo Street, Bolton BL1 2SP, UK Tel: 01204 871001

E-mail: sales@boltongate.co.uk Web: www.boltongate.co.uk

Issue Date: April 2018