FIREROLL E30/60 FIRE RESISTING ROLLER SHUTTERS

SPECIFICATION 3-3

Fire Resistance

Tested to EN 1634-1 and achieved a fire rating of E60 in accordance with EN 13501-2.

Curtain

Constructed from 51mm flat section galvanised steel laths with pressed steel endlocks fitted to alternate laths to prevent lateral movement.

Bottom Rail

Formed from a reinforced lath.

## Guides

The side guides are formed from galvanised steel channels and prepared for fixing to the opening structure by means of continuous fixing angles.

## Endplates

Fabricated from galvanised mild of adequate thickness relative to door size and supplied with angles for fixing to the structure.

## Roller

The roller is manufactured from mild steel tube of sufficient diameter and wall thickness to resist deflection and is mounted in bearings or cups on each end plate. A galvanised steel casing is supplied as standard.

Finish

Galvanised as standard.  They are also available in stainless steel and polyester powder coated in a range of standard RAL colours at extra cost.

## Weight

Approximately 25 kgs per m².

Operation

Electrical operation is by means of a 240-volt single phase Speedsafe motor mounted on one endplate.  The standard controls are by means of open / close / stop push buttons fitted adjacent to the shutter. Other types of control are available at extra cost.

Fire Activation

In a fire condition, activation is initiated by an incoming signal from the fire alarm system.  Closure is by gravity with a controlled descent.

To specify state:

Fireroll E30/60 Fire Resisting Roller Shutters shall be by

Bolton Gate Company, Waterloo Street, Bolton BL1 2SP, UK Tel: 01204 871001

E-mail: sales@boltongate.co.uk Web: www.boltongate.co.uk

Issue Date: April 2018