FIREROLL ACOUSTIC SERVERY ROLLER SHUTTERS

SPECIFICATION 3-12

Fire Resistance

Tested to BS 476 Part 22 and EN 1634-1 for two hours integrity and radiation resistance.

Acoustic Rating

When independently tested, the door achieved a weighted sound reduction index Rw of 25dB.

Curtain

Constructed from 100mm flat section galvanised steel laths with acoustic / fire resisting infill and steel endlocks fitted to alternate laths to prevent lateral movement. The bottom rail is a lath fitted with a galvanised steel section trim.

## Guides

The side guides are formed from galvanised steel channels and are prepared for fixing to the opening structure by means of continuous fixing angles.

## Endplates

Fabricated from mild steel of adequate thickness relative to door size and supplied with angles for fixing to the structure.

## Roller

The roller is manufactured from mild steel tube of sufficient diameter and wall thickness to resist deflection and is mounted in bearings on each end plate.

Finish

Galvanised as standard.  A factory finish is available which comprises a white plastisol internal face (coil side) with the external face polyester powder coated to a standard RAL colour.

## Weight

Approximately 50 kgs per m².

Operation

All shutters are electrically operated and have adjustable limit switches incorporated to stop the shutter at the end of each travel. Smaller shutters are supplied with a single phase tubular motor with larger units being driven by the highly reliable Speedsafe motor which allows gravity failsafe closure in fire conditions via a fusible link or auto reset solenoid or magnet.  The standard method of control is by means of internal wall-mounted push button control.  Other types of control including keyswitches etc. are available at extra cost.

Casings

A galvanised steel coil casing is provided to cover the roller and provide a fire seal at the head.

To specify state:

Fireroll Acoustic Servery Roller Shutters shall be by

Bolton Gate Company, Waterloo Street, Bolton BL1 2SP, UK Tel: 01204 871001

E-mail: sales@boltongate.co.uk Web: www.boltongate.co.uk

Issue Date: April 2018