BI-FOLD GATES

SPECIFICATION 17-5

Gate Leaves

The gate leaves are constructed from a 60 x 40mm RHS perimeter frame with 25mm diameter tubes fitted vertically at 125mm centres.  A mid-rail is supplied for strengthening purposes.

## Top Track

Galvanised steel inverted channel section in which pulleys mounted on each leading edge run in and on.

## Bottom Support

The gates are supported on floor bearings mounted at the side behind each jamb.

Door Arrangements

The standard arrangement comprises two leaves each side folding at 90 degrees to the opening.

Ironmongery

Handles are attached to each leading edge to allow manual operation in the event of power failure.

Finish

Galvanised as standard.  Polyester powder coating to a range of standard RAL colours is available at extra cost.

## Weight

Varies dependant on gate construction but typically 40 kgs per m2.

Operation

Electrical operation is via two single phase motors mounted on the top of each pair of leaves with a common control panel and operating buttons.  Disconnection to manual operation is provided in the event of power failure.  Other controls are available at extra cost.

To specify state:

Bi-Fold Gates shall be by

Bolton Gate Company, Waterloo Street, Bolton BL1 2SP, UK Tel: 01204 871001

E-mail: sales@boltongate.co.uk Web: www.boltongate.co.uk

Issue Date: April 2018