BOTTOM ROLLING COLLAPSIBLE GATES

SPECIFICATION 2-2

Gate Frame

Constructed from double channel rolled pickets 16mm wide x 2.5mm thick interconnected by 6mm thick galvanised steel solid lattice bars.  Ball bearing pulleys are attached at the bottom and the pickets extended at the top between which the top track runs.

Leading Edge

The leading edge is formed from two channel pickets onto which the handles are fitted.  Totally enclosed fabricated leading edges into which a lock can be mortised are available at extra cost.

## Top Track

The top track is formed from a 12mm thick solid steel tee section 50 x 50mm overall.

## Bottom Track

The bottom track is formed from a solid steel flat bar 16mm thick x 50mm high on which the bottom pulleys run.  The track is folding or portable dependant on width / height ratio or practicality.

Finish

Galvanised as standard.  Polyester powder coating to a range of standard RAL colours and stainless steel are available at extra cost.

## Weight

Approximately 32 kgs per m².

Operation

Manual operation via bow handles attached to each leading edge.

Locks

Locking is by client’s padlock. A five lever deadlock mortised into the leading edge is available at extra cost.

To specify state:

Bottom Rolling Collapsible Gates shall be by

Bolton Gate Company, Waterloo Street, Bolton BL1 2SP, UK Tel: 01204 871001

E-mail: sales@boltongate.co.uk Web: www.boltongate.co.uk

Issue Date: April 2018