TOP HUNG COLLAPSIBLE GATES

SPECIFICATION 2-1

Gate Frame

Constructed from double channel rolled pickets 16mm wide x 2.5mm thick interconnected by 6mm thick galvanised steel solid lattice bars.  Ball bearing pulleys are attached at the top and zinc-plated guide pieces at the bottom.

Leading Edge

The leading edge is formed from two channel pickets onto which the handles are fitted.  Totally enclosed fabricated leading edges into which a lock can be mortised are available at extra cost.

## Top Track

Galvanised steel rolled section forming a box-type track 90mm deep.

## Bottom Track

The bottom track is formed from a 26 x 25mm galvanised steel channel recessed into the floor.  Folding or portable bottom tracks are available at extra cost should it be preferable to have an unbroken floor surface.

Finish

Galvanised as standard.  Polyester powder coating to a range of standard RAL colours and stainless steel are available at extra cost.

## Weight

Approximately 32 kgs per m².

Operation

Manual operation via bow handles attached to each leading edge.

Locks

Locking is by client’s padlock. A five-lever deadlock mortised into the leading edge is available at extra cost.

To specify state:

Top Hung Collapsible Gates shall be by

Bolton Gate Company, Waterloo Street, Bolton BL1 2SP, UK Tel: 01204 871001

E-mail: sales@boltongate.co.uk Web: www.boltongate.co.uk

Issue Date: April 2018