UNIGUARD HINGED STEEL DOORS

SPECIFICATION 6-2

Door Arrangements

Doors can be supplied as singles, pairs, leaf and a half with or without side and transom panels.

Door Leaves

Manufactured from 1.2mm thick galvanised steel sheets stiffened with internal honeycomb material and reinforced for hardware.  The overall panel thickness is 43mm and has no face seams.

## Frame

Manufactured from 1.5mm galvanised steel and formed into a single rebate profile.  Supplied in either knockdown or fully welded form.  The frames are morticed, reinforced, drilled and tapped for hinges and strike plates.

Finish

Prime painted for finishing on site by others.  Polyester powder coating to a range of standard RAL colours is available at extra cost.

## Weight

Approximately 35 kgs per m².

Hardware

Various items of hardware can be supplied / accommodated.  A typical set to be specified as a minimum would comprise:

1½ pairs of stainless steel hinges per leaf

Mortice lock / latch

Lever handles

Medium duty door closer

Flush bolts to the inactive leaf of pairs

To specify state:

Uniguard Hinged Steel Doors shall be by

Bolton Gate Company, Waterloo Street, Bolton BL1 2SP, UK Tel: 01204 871001

E-mail: sales@boltongate.co.uk Web: www.boltongate.co.uk

Issue Date: April 2018